SN24.4/10-98

DR655xx/DR69000

Display Driver Package

Release Notes for Windows NT 4.00

Revision:	Version 1.2.9
Release Date:	September 9, 1998

- Supported 100 Hz refresh in DirectDraw.

- Added 100 Hz refresh.

- DrvCopyBits was incorrectly processing operations where Source was the video memory and Destination was a bitmap in system memory (Screen->System bitmap transfer).

The following enhancements for 65545 and 65548 have been made:

- 65548.  When running on NT 4.0 Japanese system, switch from Windowed to Full Screen DOS session did not work properly, it has been corrected.

- 65545/65548.  Switch to 24bpp mode was allowed when in Simultaneous/LCD display mode.

This has been changed to allow 24bpp modes only when CRT display mode is selected.

Revision:	Version 1.1.7
Release Date:	March 11, 1998

The following problems have been fixed in the current release:

- 1280x1024x256 Interlaced mode selection was not available.

Enhancements:

- 69000 chip support has been added with this release.  This includes support for 69000 NTSC/PAL Vertical Reduction (Underscan Scaling) which has been added to Chips Control Panel Tab and display driver.

Revision:	Version 1.1.5
Release Date:	December 15, 1997

The following problems have been fixed in the current release:

1.	When running DDTEST, and then opening and closing Active Movie sessions, an “OUTOFVIDEOMEMORY” error would be reported.  This was due to the failure of ddraw to release offscreen video memory used for Overlay buffer when Active Movie session was closed.

- WHQL direct draw Overfly test when run in Simultaneous mode would receive garbage in right side of overlay rectangle when the screen is panned to the right.

- Tracker #346.  Dual Processor systems would hang when moving large Windows around the desktop.

Enhancements:

NTSC/PAL mode panning enable/disable have been added to Chips Control Panel Tab.

Revision:	Version 1.1.3
Release Date:	September 19, 1997

The following enhancements have been made:

- Direct Draw Hardware Overlay support has been enabled for 6555x adapters.

- Installation of CHIPS Control Panel Tab has been added to driver install.

- Enable/Disable of NTSC/PAL video mode panning has been added to CHIPS Control Panel Tab.

The following problems have been fixed in the current release:

- When playing an MPEG file and panning in mode 1280x1024 and 1024x768 colors, the color key seeps from the right.

- Graphics Stretching does not stretch the direct draw window.

- NTSC/PAL video modes do not align the video window properly.

- NTSC/PAL  video playback is not being forced to an even start line.

- Switching between NTSC/PAL video mode and CRT/SIMUL/LCD modes causes corruption in Active Movie window.

- Switching from NTSC/PAL mode to LCD mode using CHIPS Control Panel Tab shifts image to upper part of screen, with lower portion filled with garbage.  ( Tracker  #508 )

- NTSC/PAL video mode switch to DOS Full Screen has incorrect font.

- 65550 systems that are configured to support NTSC/PAL video would corrupt the display when running 64K Color modes, and switching from DOS Window to DOS Full Screen.

- Cursor disappears after return from video mode test.  ( Tracker #499)

- Blue Screen when running EDIT in DOS Window and then switching to Full Screen.

- Invalid Video Modes presented for selection in Display Settings when system is switched to Simultaneous Display Mode. (Tracker #262)

- Systems with 65535/65540 adapters hang during boot.

Revision:	Version 1.0.8
Release Date:	May 22, 1997

The following enhancements have been made:

- Added support for NTSC/PAL CRT Panning.

- Added support for 65555 and 68554 chips.

The following problems have been fixed in the current release:

- Pre-install of driver does not work.  Defaults to VGA.

- 65548 - Cannot switch to Full Screen DOS session.

- 65548 - When in Display Settings, selecting Test to test new display setting causes system to hang.

Failures 1 and 2 are due to a problem with BIOS Set Refresh function, which was introduced in later versions of 65548 Video BIOS.  Fix has been temporarily made in miniport driver until BIOS fix is available.

- 65545/548 VL/ISA bus systems - The system will hang when doing a Restart after shutdown.

- When switching to NTSC video, two cursor images appear on screen.

- Cursor movement and position is not correct when Graphics Mode stretching is enabled.  Switch to software cursor is now done to allow proper cursor movement.

- Fixed NT 4.0 J problem switching to DOS fullscreen session.

- Underline Text Cursor is missing after switching from Desktop to DOS Command Prompt fullscreen session.

- Text Expansion gets reset during restore of DOS Command Prompt fullscreen session.

- Invalid refresh selection (0 Hz) listed in Display Settings.

- Fixed problem with corruption in display window when using Media Player to play and MPEG (.MPG) file.

- Fixed problem with AVI window freezing when playing an overlapping DOS Command Prompt window was switched to fullscreen, then switched back to a window, and then minimized to tak bar.  Media Player control would indicate that the video clip was playing, but nothing was being drawn in AVI window.

- Fixed problem with 24bpp Transparent SRC/DST bitblt operations in Direct Draw.

- Modified code to correctly output large font (glyphs) that reside in “packed” font files.

- Full screen graphic VDMs are stretched vertically and horizontally.

- Mouse cursor disappears when you go to the right edge or bottom edge in 800x600 or 1024x768 modes.

- Start WinNT 4.0 with true color mode (mode 50 or 52).  Select any background pattern from the Display Properties Background sheet.  Click OK.  You will see that the background palette is corrupt.

- Memory size detection incorrect on 65554 which caused display modes to be excluded from selection.