************************************************************************

*             How to compile and link C or C++ application             *

*             program with the PCM-3718 Device Driver                  *

************************************************************************

1. How to compile and link with the driver for Microsoft C  ?

Example for SMALL mode :

cl  /AS  /c  3718demo.c

link 3718demo + 3718CS.LIB;

Note :    /AS for SMALL Mode

/AM for MEDIUM Mode

/AC for COMPACT Mode

/AL for LARGE Mode

2. How to compile and link with the driver for Turbo C  ?

Example for SMALL mode :

1. In Integrated Development Environment

. Set project name (*.prj) in project options

Example : 3718.prj

. 3718.prj file contents :

3718demo.c

3718cs.lib

. Select Compile \ Build all

2. In DOS Command Use TCC Program

. TCC   -ms  3718demo.c  3718cs.lib

Note :    -ms for SMALL Mode

-mm for MEDIUM Mode

-mc for COMPACT Mode

-ml for LARGE Mode

3. How to compile and link with the driver for Borland C++  ?

Example for SMALL mode :

BCC   -ms 3718demo.c 3718cs.lib

Note :    -ms for SMALL Mode

-mm for MEDIUM Mode

-mc for COMPACT Mode

-ml for LARGE Mode

************************************************************************

*  If linker error, undefined symbol pcm3718(int, unsigned int far*),  *

*  occurred when using .CPP as program extension name                  *

************************************************************************

1. Add the following code paragraph before the declaration

extern pcm3718(int, unsigned int *):

#ifdef __cplusplus

extern "C" {

#endif

_cdecl pcm3718(int, unsigned int *);

#ifdef __cplusplus

}

#endif